Hazard and Operability Study (HAZOP)

# HAZOP 1

HAZOP 1 (Preliminary Hazard Review) is done at the very early stage of any project or modification to identify safety health & environmental aspects of the materials in the process, which need further attention. This provides information for compiling the Environmental Impact Assessment.

## Project description

The proposed project is a demonstration project to prove biodiesel can be produced from Used Cooking Oil in accordance with the specifications of SANS1935; as currently being done in the EU and US. Producing high quality and compliant biodiesel from used cooking oil is more challenging than producing it from virgin vegetable oils. Therefore, when proven that the biodiesel can be produced from used cooking oil in compliance with SANS1935, this should give credibility to biodiesel projects which use vegetable oils as well. This project will therefore improve the standard of biodiesel production in South Africa and familiarize the market to biodiesel. Additionally, this project will be developed as a community project as the project aims to sell its biodiesel at discounted prices to local Daveyton Township and surrounds.

The business objectives of the venture is to:

- demonstrate its commercial feasibility within a year of operation,

- identify opportunities to refine the technology,

- determine a commercialisation approach, and

- be poised for next stage growth and development

The objective of this project is to design, construct, commission, package, and operate a small two step demonstration scale biodiesel plant that uses waste cooking oil to:

- produce on spec biodiesel (SANS1935) by January 2016 	(Prove Chemistry)

- produce at a rate of 5400ltr/day for 3 months 		(Prove Plant Performance)

- produce min of 300kLtr until Jun 2016 			(Meet Financial Goals)

## Process description

The two step biodiesel process is essentially a multi-step batch process.

Figure 1: Biodiesel process steps

Tankage:

- Feeds (waste cooking oil, methanol, KOH, H2SO4, additives) are stored onsite

- Intermediate products might be stored or buffered onsite

- Products (diesel, glycerol) are stored onsite

Filtration:

- UCO is filtrated prior to storage and prior to entering the plant

- Intermediate & final filtration might also be required

Acid Treatment:

- The waste cooking oil contains 5-30% fatty acids which are esterified using excess methanol and H2SO4 as a catalyst.

- Small quantities of water (1-5%) inhibits forward reaction equilibrium

- Higher quantities of fatty acids might require a two-step esterification

- Residual fatty acids will result in soap formation in the Base Treatment step, resulting in diesel yield loss and separation difficulty in subsequent process steps

- Process conditions: typically 50-70°, atmospheric, 1-2 hours, intense mixing required

- If a wash or neutralisation is required the process step produces and methanol + water (possibly glycerol) and salts effluents respectively which needs workup or recycled.

Base Treatment:

- The acid mixture is neutralised using and alkalinity increased using KOH (aq)

- Excess methanol + KOH is added and reacts in a transesterification reaction to produce esters and glycerol

- Two step trans-esterification might be required to reach conversion targets

- Process conditions: typically 50-70°, atmospheric, 1-2 hours, high mixing required

- Glycol + Water + methanol + salt effluent are produced which needs workup

Separation and washing:

- Diesel (ester product) separate physically from glycerol + water + methanol layer

- Glycerol + water + methanol layer are drained and sent to effluent workup for glycerol refining, and methanol and water and recycling

- Diesel is washed with water to remove residual glycerol, salt and methanol: washing liquid is drained and sent to effluent workup.

- Process conditions: typically 50-70°, atmospheric, 1-2 hours

Diesel Polishing:

- Diesel is polished using a ion exchange resin (removes traces of methanol, glycerol, water, salts and catalyst)

- Ion exchange resin is regenerated using methanol and produced contaminated diesel and methanol in the process which needs recycling or workup.

Effluent workup and Recycling:

- Different effluents are generated in the process, some of which can be recycled directly and others require different degrees of workup.

- Glycerol refining: Methanol and water content is reduced to upgrade the glycerol

- Methanol and water are separated and recycled

## Chemical hazard assessment

It is necessary to evaluate each chemical present in the plant’s processes to ensure that all hazards associated with the different chemicals are identified.

For the Chemical Hazard Guide Diagram, refer to Appendix A.

Table 1: Chemical Hazard Data

|  | Chemical or Group of Chemical | Physical State | Quantity (Inventory or through-put) |
| --- | --- | --- | --- |
| A | Used Cooking Oil | Liquid | ± 5500 L |
| B | Sulphuric Acid (H2SO4) | Liquid | ± 220 L |
| C | Methanol | Liquid | ± 1000 L |
| D | Potassium Hydroxide | Pellets | ± 50 kg |
| E | Potassium Methoxide | Liquid | ± 96 L |
| F | Sodium Hydroxide | Pellets / Flakes | ± 50 kg |
| G | Sodium Methoxide | Powder | ± 96 L |
| H | Ionol | Liquid | ± 5.03 L |
| I | Viscoplex | Liquid | ± 10.05 L |
| J | Biodiesel | Liquid | ± 1675 L |
| K | Glycerol | Liquid | ± 250 L |

Table 2: Chemical Hazard Information

| Hazard Potential ‘-’ Key ‘K’ No. | Insignificant Hazard Hazards known and understood See numbered notes |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I | J | K |
| Explosion and Flammability Hazards |  |  |  |  |  |  |  |  |  |  |  |
| Fire | K | K | K | - | K | - | K | K | - | K | - |
| Deflagration/ Detonation | - | K | K | - | - | - | - | - | - | K | - |
| Electrical Static | - | K | K | - | K | - | K | - | - | K | - |
| Reactivity/Stability Hazards | - | K | K | - | K | - | K | K | - | K | - |
| Immediate Health Hazards |  |  |  |  |  |  |  |  |  |  |  |
| Inhalation Toxicity | - | K | K | K | K | K | K | K | - | K | K |
| Skin Absorption | - | K | K | K | K | - | K | - | - | K | - |
| Corrosive | - | K | - | K | K | K | K | - | - | - | - |
| Chronic Health Hazards |  |  |  |  |  |  |  |  |  |  |  |
| Digestive | K | K | K | K | K | K | K | K | - | K | K |
| Sensitiser | - | - | - | - | - | - | - | - | - | - | - |
| Continual | - | K | - | K | - | - | - | K | - | K | - |
| Other Health Hazards |  |  |  |  |  |  |  |  |  |  |  |
| Odour | K | - | K | - | K | - | K | - | K | K | K |
| Radiation | - | - | - | - | - | - | - | - | - | - | - |
| Environmental Hazards |  |  |  |  |  |  |  |  |  |  |  |
| Aqueous | K | K | K | K | - | K | - | K | - | K | K |
| Gaseous | - | - | - | - | - | - | - | - | - | - | - |
| Ground | K | - | - | - | - | K | - | - | - | - | - |
| Hazard Breakdown Products | K | K | K | K | K | K | K | K | - | K | K |

## Chemical interactions assessment

It is necessary to evaluate the interactions between chemicals in order to identify possible hazards that can emerge because of these interactions. The interaction between chemicals and the construction materials that are in direct contact with these process chemicals need to be evaluated to ensure that the correct construction materials are chosen.

Table 3: Chemical Interactions

| Hazard Potential '-' Insignificant Hazard Key 'K' Hazards known and understood No. (e.g. 1,2,3...) See numbered notes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chemical or Group of Chemicals |  |  | B | C | D | E | F | G | H | I | J |  | K |
| A |  | A | K | K | - | - | - | - | - | - | - |  | - |
| B |  |  | B | K | K | - | K | - | - | - | - |  | K |
| C |  |  |  | C | K | K | K | K | - | K | - |  | - |
| D |  |  |  |  | D | - | - | - | - | - | - |  | - |
| E |  |  |  |  |  | E | - | - | - | - | - |  | - |
| F |  |  |  |  |  |  | F | - | - | - | - |  | - |
| G |  |  |  |  |  |  |  | G | - | - | - |  | - |
| H |  |  |  |  |  |  |  |  | H | - | - |  | - |
| I |  |  |  |  |  |  |  |  |  | I | - |  | - |
| J |  |  |  |  |  |  |  |  |  |  |  | J | - |
| K |  |  |  |  |  |  |  |  |  |  |  |  | K |
| Chemical or Group of Chemicals |  | A | B | C | D | E | F | G | H | I | J |  | K |
| Material of Construction |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Means of handling

For the Means of handling Guide diagram, refer to Appendix A.

Table 4: Means of Handling

|  | A | B | C | D | E | F | G | H | I | J | K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Storage | K | K | K | K | K | K | K | K | K | K | K |
| Transport | - | - | - | K | - | K | - | - | - | K | - |
| Materials Handling | K | K | K | K | K | K | K | K | K | K | K |
| Extreme Process Conditions | K | K | K | K | K | K | K | - | K | K | - |
| Other Construction Materials | - | K | K | K | - | K | - | - | - | - | - |
| Decontamination | K | K | K | K | K | K | K | K | K | K | K |
| Gaseous Emissions | - | - | - | K | K | K | K | - | - | - | K |
| Liquid Releases | K | K | K | K | K | K | K | - | K | K | K |
| Disposal of Wastes | K | K | K | K | K | K | K | K | K | K | K |
| Destruction of Material | - | K | K | - | K | - | - |  | K | - | - |
| Quality Control | - | - | - | - | - | - | - | - | - | - | - |
| Emergency Procedures | K | K | K | K | K | K | K | K | K | K | K |
| Plant Layout, Spacing, Access | K | K | K | K | K | K | K | K | - | K | - |
| Area Classification | K | K | K | K | K | K | K | K | - | K | K |
| Provision of Services | - | K | K | K | K | K | K | K | K | K | K |
| Codes of Practice | - |  |  |  |  |  |  |  |  |  |  |

## Environmental statement

## Site selection and plant layout

The proposed demonstration project is to be constructed and operated at 110 Peenz St, Putfontein, Benoni, Gauteng, South Africa (26°06’13.10”S; 28°22’25.05”S). See Appendix B for site maps and pictures. The plant will be situated approximately 4 km from the Daveyton Township and Zenzele informal settlement. The need for clean, more affordable fuels, as well as jobs and economic development are high in this poverty stricken township and therefore the project is foreseen to have a high direct impact by being closely allocated. A biodiesel fuelling station will be installed at the project site to provide access to local community and 4 entrepreneurs will be identified and empowered to take the biodiesel into the local community for sales using a truck and tank, and also assist on the logistics on raw materials. The biodiesel will be marketed to the township by means of posters and flyers to be distributed in local community centres, schools, etc.

The facility will be located on a site which will be large enough to accommodate a tank farm and warehouse for the process equipment.

Figure 2: Site location

The site will be limited to store 30 000 litre flammable liquid. This includes feedstock and product which will be stored in a tank farm on site.

- The site should have sufficient access and infrastructure for loading and unloading of equipment, feedstock, and product. Receiving and dispatching from tanker trucks possible.

- Care should be taken to consider aspect such as fire protection, bunding and explosion circles prior to selecting a site.

- The facility will have a footprint of approximately 300 m2 (12 x 25 m)

- Tank farm area will require approximately 150 m2

The following utilities and infrastructure are available:

- Potable water: The site is coupled to the domestic water supply

- Sewer: Local French-drain system

- Electricity: Three phase, 80 amp Max

- Road and Access: The site is easily accessed via good quality asphalt roads.

- Logistics: Turnaround circles can be established for large vehicles if required

- Offices: Offices are available on site

- Amenities: Toilets and kitchen are available on site

- Buildings: A 25 m x 12 m steal frame and zinc IBR roofed industrial building with concrete floors and sliding doors is available

- Workshops: The building is large enough to include a small workshop function

- Storage: The building is large enough to include a small store. Additional storage rooms available on site.

- Alarm and Security: The site is situated in a lager complex which has gate security and shrouding fencing. Security fencing around tank farm will be required and an alarm system will be needed.

- Laboratory: A small routine analytical laboratory is required established

- Fire system: No fire protection infrastructure exists

- Storm Water System: Storm water runs down and collect in a pond 10m next to the building

- Tankage: No tankage area exist currently. It is foreseen that open area at the back of the building will be utilised as a tank farm. Civil work, including concrete foundation, bunding and fire isolation is foreseen.

Figure 3: Plant layout

# HAZOP 2

This HAZOP includes the identification of significant hazards that are associated with plant items and equipment.

For Guide Diagrams 1 and 2, refer to Appendix A.

Table 1: HAZOP 2

| PLANT ITEM DESCRIPTION | REACTORS |  |  |  |  |  | FILTERS / PURIFIERS |  |  |  | MIXERS |  |  |  |  | STORAGE TANKS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POSSIBLE? TICK YES COMPLETE WORKSHEET | NO |  | YES |  | W/S NO: |  | NO | YES |  | W/S NO: | NO |  | YES |  | W/S NO: | NO | YES | W/S NO: |  |
| EXTERNAL FIRE |  |  | X |  | 01 |  |  | X |  | 10 |  |  | X |  | 17 |  | X | 27 |  |
| INTERNAL FIRE (In equipment) |  |  | X |  | 02 |  | X |  |  |  |  |  | X |  | 18 |  | X | 28 |  |
| INTERNAL EXPLOSION (In equipment) | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  | X | 29 |  |
| CONFINED EXPLOSION (In building/structure) | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| UNCONFINED EXPLOSION (Outside in open) | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| INTEGRITY LOSS |  |  | X |  | 03 |  |  | X |  | 11 |  |  | X |  | 19 |  | X | 30 |  |
| ACUTE HARMFUL/ NOXIOUS EXPOSURE |  |  | X |  | 04 |  | X |  |  |  |  |  | X |  | 20 |  | X | 31 |  |
| ACUTE HARMFUL/ PHYSICAL EXPOSURE |  |  | X |  | 05 |  | X |  |  |  |  |  | X |  | 21 |  | X | 32 |  |
| CHRONIC HARMFUL/ NOXIOUS EXPOSURE |  |  | X |  | 06 |  |  | X |  | 12 |  |  | X |  | 22 | X |  |  |  |
| SPILLS | X |  |  |  |  |  |  | X |  | 13 |  |  | X |  | 23 | X |  |  |  |
| POLLUTION |  |  | X |  | 07 |  |  | X |  | 14 |  |  | X |  | 24 |  | X | 33 |  |
| NOISE | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| VIOLENT RELEASE OF ENERGY | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  | X | 34 |  |
| MOMENTUM CHANGE | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| VISUAL IMPACT | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| UNAVAILABILITY |  |  | X |  | 08 |  |  | X |  | 15 |  |  | X |  | 25 |  | X | 35 |  |
| SECURITY BREACH | X |  |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| FINANCIAL PROBLEM |  |  | X |  | 09 |  |  | X |  | 16 |  |  | X |  | 26 |  | X | 36 |  |
| PLANT ITEM DESCRIPTION |  | PUMPS |  |  |  |  | HEATERS |  |  |  | COMPRESSOR |  |  |  |  | COLUMNS |  |  |  |
| POSSIBLE? TICK YES COMPLETE WORKSHEET |  | NO |  | YES |  | W/S NO: | NO |  | YES | W/S NO: | NO | YES |  | W/S NO: |  | NO | YES |  | W/S NO: |
| EXTERNAL FIRE |  |  |  | X |  | 37 |  |  | X | 50 |  | X |  | 59 |  |  | X |  | 71 |
| INTERNAL FIRE (In equipment) |  |  |  | X |  | 38 |  |  | X | 51 |  | X |  | 60 |  | X |  |  |  |
| INTERNAL EXPLOSION (In equipment) |  |  |  | X |  | 39 |  |  | X | 52 |  | X |  | 61 |  | X |  |  |  |
| CONFINED EXPLOSION (In building/structure) |  | X |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| UNCONFINED EXPLOSION (Outside in open) |  |  |  | X |  | 40 | X |  |  |  |  | X |  | 62 |  | X |  |  |  |
| INTEGRITY LOSS |  |  |  | X |  | 41 |  |  | X | 53 |  | X |  | 63 |  |  | X |  | 72 |
| ACUTE HARMFUL/ NOXIOUS EXPOSURE |  |  |  | X |  | 42 |  |  | X | 54 |  | X |  | 64 |  | X |  |  |  |
| ACUTE HARMFUL/ PHYSICAL EXPOSURE |  |  |  | X |  | 43 |  |  | X | 55 |  | X |  | 65 |  | X |  |  |  |
| CHRONIC HARMFUL/ NOXIOUS EXPOSURE |  |  |  | X |  | 44 | X |  |  |  |  | X |  | 66 |  | X |  |  |  |
| SPILLS |  |  |  | X |  | 45 | X |  |  |  |  | X |  | 67 |  | X |  |  |  |
| POLLUTION |  | X |  |  |  |  |  |  | X | 56 | X |  |  |  |  |  | X |  | 73 |
| NOISE |  |  |  | X |  | 46 | X |  |  |  |  | X |  | 68 |  | X |  |  |  |
| VIOLENT RELEASE OF ENERGY |  |  |  | X |  | 47 | X |  |  |  | X |  |  |  |  | X |  |  |  |
| MOMENTUM CHANGE |  | X |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| VISUAL IMPACT |  | X |  |  |  |  | X |  |  |  | X |  |  |  |  |  | X |  | 74 |
| UNAVAILABILITY |  |  |  | X |  | 48 |  |  | X | 57 |  | X |  | 69 |  |  | X |  | 75 |
| SECURITY BREACH |  | X |  |  |  |  | X |  |  |  | X |  |  |  |  | X |  |  |  |
| FINANCIAL PROBLEM |  |  |  | X |  | 49 |  |  | X | 58 |  | X |  | 70 |  |  | X |  | 76 |

| HAZARD & OPERABILITY STUDY 2 RECORD |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Ref. no: | Hazardous event | Causes | Consequences | Preventative measures | Protective measures | Recommendation |
| REACTORS |  |  |  |  |  |  |
| 01 | External fire | - Flammable material - Loss of containment - Auto ignition | - A, E, F, G | - Regular inspections and maintenance - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 02 | Internal fire | - Flammable material - Loss of containment - Auto-ignition - Static | - A, C, E, F, G, J, M | - Regular inspections and maintenance - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 03 | Integrity loss | - Refractory erosion - Reactor jet damage - Cracking - Thermal stress | - A, C, E, F, G, J, M | - Regular inspections and maintenance | - Material of construction |  |
| 04 | Acute harmful/ noxious exposure | - Toxic material - Loss of containment | - A, M | - Regular inspections and maintenance | - PPE |  |
| 05 | Acute harmful/ physical exposure | - Hot surfaces - Leakage | - A, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 06 | Chronic harmful/ noxious exposure | - Toxic material - Leakage | - B, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 07 | Pollution | - Loss of containment | - J, L | - Regular inspections and maintenance |  | - Quality transmitter and control |
| 08 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| 09 | Financial problems | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace or damaged equipment |  |
| FILTERS / PURIFIERS |  |  |  |  |  |  |
| 10 | External fire | - Flammable material - Loss of containment - Auto ignition | - A, E, F, G | - Regular inspections and maintenance - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 11 | Unconfined explosion | - Corrosion - Flammable material | - A, C, E, F, G, J, M | - Regular inspections and maintenance | - PPE - Keep spare parts on site for fast repairs | - Temperature transmitters |
| 12 | Acute harmful/ Physical exposure | - Hot surface - Over pressure effects | - A, M | - Access control - Insulation | - PPE - First aid kit | - Visual warning signs - Awareness |
| 13 | Chronic harmful/ Noxious exposure | - Leaks or bursts | - A, C, E, F, G, J, M | - Monitoring flow rates and pressure drops - Regular inspections and maintenance | - Clean contaminated areas as soon as possible | - Supply means of handling diagram |
| 14 | Spills | - Leaks, bursts | - A, C, E, F, G, J, M | - Monitoring flow rates and pressure drops - Regular inspections and maintenance | - Clean contaminated areas as soon as possible | - Supply means of handling diagram |
| 15 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| 16 | Financial problem | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| MIXERS |  |  |  |  |  |  |
| 17 | External fire | - Flammable material - Loss of containment - Auto ignition | - A, E, F, G | - Regular inspections and maintenance - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 18 | Internal fire | - Flammable material - Loss of containment - Auto-ignition - Static | - A, C, E, F, G, J, M | - Regular inspections and maintenance - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 19 | Integrity loss | - Refractory erosion - Cracking - Thermal stress | - A, C, E, F, G, J, M | - Regular inspections and maintenance | - Material of construction |  |
| 20 | Acute harmful/ noxious exposure | - Toxic gasses - Loss of containment | - A, M | - Regular inspections and maintenance | - PPE |  |
| 21 | Acute harmful/ physical exposure | - Hot surfaces - Heater tube burst - Leakage | - A, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 22 | Chronic harmful/ Noxious exposure | - Toxic material - Leakage | - B, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 23 | Spills | - Leaks, bursts | - A, C, E, F, G, J, M | - Monitoring flow rates and pressure drops - Regular inspections and maintenance | - Clean contaminated areas as soon as possible | - Supply means of handling diagram |
| 24 | Pollution | - Loss of containment | - J, L | - Regular inspections and maintenance |  |  |
| 25 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| 26 | Financial problems | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| STORAGE TANKS |  |  |  |  |  |  |
| 27 | External fire | - Flammable material - Loss of containment | - A, E, F, G, J, M | - Regular inspections and maintenance - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan - Flammable material should be separated and stored 4m away from other material |
| 28 | Internal fire | - Flammable gas | - F, G | - Regular inspections and maintenance - Monitor temperatures |  | - Temperature alarms |
| 29 | Internal explosion | - Flammable gas - Vessel over pressure | - A, C, E, F, G, J, M | - Regular inspections and maintenance - Monitor temperatures and pressures | - Proper ventilation |  |
| 30 | Integrity loss | - Erosion - Corrosion | - A, C, E, F, G, J, M | - Adequate material of construction - Regular inspections and maintenance - Monitor compositions | - Replace or repair damaged equipment |  |
| 31 | Acute harmful/ Noxious exposure | - Toxic gasses - Loss of containment | - A, M | - Regular inspections and maintenance - Monitor flow rates - Access control | - PPE | - Flow alarms |
| 32 | Acute harmful/ physical exposure | - Leakage | - A, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 33 | Pollution | - Loss of containment | - J, L | - Regular inspections and maintenance |  | - Quality transmitter and control |
| 34 | Violent release of energy | - Electrical - electrostatic | - A, C, E, F, G, J, M | - Store materials at least 4m from any electricity | - PPE - First aid kit |  |
| 35 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| 36 | Financial problems | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace or damaged equipment |  |
| PUMPS |  |  |  |  |  |  |
| 37 | External fire | - Loss of containment, flammable material can ignite with sparks | - A, E, F, G | - Do not operate unit if leakage is detected - Regular inspections - Do not smoke near unit - Monitoring the equipment frame and liquid end temperature. - Maintaining proper bearing lubrication. - Ensuring that the pump is operated in the intended hydraulic range. | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 38 | Internal fire | - Flammable gas, sparks within compressor, friction, hot spots, auto ignition | - F, G | - Do not install near an open flame or furnace - Maintenance | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 39 | Internal explosion | - Overpressure - Flammable gas - Restricted ventilation | - A, C, E, F, G, J, M | - Ensure correct system components are used - Ensure proper ventilation - Regular inspections and maintenance | - PPE - Fire alarms - Fire fighting equipment - Replace damaged equipment | - Backup pumps and compressors - Effective evacuation plan |
| 40 | Unconfined explosion | - Restricted ventilation | - A, C, E, F, G, J, M | - Ensure proper ventilation - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment - Replace damaged equipment | - Effective evacuation plan |
| 41 | Integrity loss | - Fatigue - Pressure | - A, C, E, F, G, J, M | - Regular maintenance - Regular inspections | - PPE - Keep spare parts on site for fast repairs | - Pressure transmitters - Backup pumps and compressors |
| 42 | Acute harmful/ Noxious exposure | - Toxic gasses - Loss of containment | - A, M | - Regular inspections and maintenance - Monitor flow rates - Access control | - PPE | - Flow alarms |
| 43 | Acute harmful/ Physical exposure | - Burst - Leakage | - A, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 44 | Chronic harmful/ Noxious exposure | - Toxic material - Leakage | - B, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 45 | Spills | - Leaks, bursts | - A, C, E, F, G, J, M | - Monitoring flow rates and pressure drops - Regular inspections and maintenance | - Clean contaminated areas as soon as possible | - Supply means of handling diagram |
| 46 | Noise | - Machinery - Pressure pulses | - J, K | - Relocate nearby employees far from noise source. - Absorptive type silencer. | - Ear protection - Relocate employees far from noise source |  |
| 47 | Violent release of energy | - Electrical arc - Earth fault | - A, C, E, F, G, J, M | - Earth pump connected to stainless steel reactor | - PPE | - Visual warning signs - Awareness |
| 48 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment | - Backup pumps and compressors |
| 49 | Financial problem | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace damaged equipment |  |
| HEATERS |  |  |  |  |  |  |
| 50 | External fire | - Flammable gas - Loss of containment - Hot spots | - A, E, F, G | - Regular inspections and maintenance - Do not operate unit if leakage is detected - Do not smoke near unit - Monitoring the equipment frame and gas end temperature. | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 51 | Internal fire | - Flammable gas - Pump failure causing higher fuel flow rate to heater | - F, G | - Regular inspections and maintenance - Monitor temperatures | - Backup pumps | - Temperature transmitter and controller to control heater temperature |
| 52 | Internal explosion | - Flammable gas - Vessel over pressure | - A, C, E, F, G, J, M | - Regular inspections and maintenance - Monitor temperatures and pressures | - Proper ventilation | - Pressure relief valves |
| 53 | Integrity loss | - Corrosion | - A, C, E, F, G, J, M | - Regular inspections and maintenance - Material of construction | - Materials of construction |  |
| 54 | Acute harmful/ noxious exposure | - Toxic gasses - Loss of containment | - A, M |  | - PPE |  |
| 55 | Acute harmful/ physical exposure | - Hot surfaces - Heater tube burst - Heater coil damage - Leak | - A, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 56 | Pollution | - Loss of containment | - J, L | - Regular inspections and maintenance |  |  |
| 57 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| 58 | Financial problems | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| COMPRESSORS |  |  |  |  |  |  |
| 59 | External fire | - Loss of containment, flammable material can ignite with sparks | - A, E, F, G | - Do not operate unit if leakage is detected - Regular inspections - Do not smoke near unit - Monitoring the equipment frame and liquid end temperature. - Maintaining proper bearing lubrication. - Ensuring that the pump is operated in the intended hydraulic range. | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 60 | Internal fire | - Flammable gas, sparks within compressor, friction, hot spots, auto ignition | - F, G | - Do not install near an open flame or furnace - Maintenance | - PPE - Fire alarms - Fire fighting equipment | - Effective evacuation plan |
| 61 | Internal explosion | - Overpressure - Flammable gas - Restricted ventilation | - A, C, E, F, G, J, M | - Ensure correct system components are used - Ensure proper ventilation - Regular inspections and maintenance | - PPE - Fire alarms - Fire fighting equipment - Replace damaged equipment | - Backup pumps and compressors - Effective evacuation plan |
| 62 | Unconfined explosion | - Restricted ventilation | - A, C, E, F, G, J, M | - Ensure proper ventilation - Do not smoke near unit | - PPE - Fire alarms - Fire fighting equipment - Replace damaged equipment | - Effective evacuation plan |
| 63 | Integrity loss | - Fatigue - Pressure | - A, C, E, F, G, J, M | - Regular maintenance - Regular inspections | - PPE - Keep spare parts on site for fast repairs | - Pressure transmitters - Backup pumps and compressors |
| 64 | Acute harmful/ Noxious exposure | - Toxic gasses - Loss of containment | - A, M | - Regular inspections and maintenance - Monitor flow rates - Access control | - PPE | - Flow alarms |
| 65 | Acute harmful/ Physical exposure | - Burst - Leakage | - A, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 66 | Chronic harmful/ Noxious exposure | - Toxic material - Leakage | - B, M | - Isolation | - PPE | - Visual warning signs - Awareness |
| 67 | Spills | - Leaks, bursts | - A, C, E, F, G, J, M | - Monitoring flow rates and pressure drops - Regular inspections and maintenance | - Clean contaminated areas as soon as possible | - Supply means of handling diagram |
| 68 | Noise | - Machinery - Pressure pulses | - J, K | - Relocate nearby employees far from noise source. - Absorptive type silencer. | - Ear protection - Relocate employees far from noise source |  |
| 69 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment | - Backup pumps and compressors |
| 70 | Financial problem | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace damaged equipment |  |
| COLUMNS |  |  |  |  |  |  |
| 71 | Internal fire |  |  |  |  |  |
| 72 | Integrity loss | - Corrosion | - A, C, E, F, G, J, M | - Adequate materials of construction |  |  |
| 73 | Pollution |  |  |  |  |  |
| 74 | Visual impact | - Building profile | - J, K |  |  |  |
| 75 | Unavailability | - Breakdown of equipment | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |
| 76 | Financial problems | - Production interruption - Major downtime | - F, G | - Regular inspections and maintenance | - Replace or repair damaged equipment |  |

# APPENDIX A

Table 5: Chemical hazard guide diagram

–

Table 6: Means of handling guide diagram - HAZOP 1

Table 7: HAZOP 2 - Guide diagram 1

Table 8: HAZOP 2 - Guide diagram 2

# APPENDIX B

Figure 4: Site from Google Maps image

Figure 5: Google Maps image of site

Figure 6: Google Maps site layout

Figure 7: Site/ Plant layout

Figure 8: Pictures of site